Revised “ADRC Core” MRI Protocol Implementation Guidance

The new protocol will go into effect Monday, October 29, 2018.

The new scan protocol is called “ADRC_Core-(10-11-2018)” and is saved on each research scanner: WIMR Premier, WIMR MR750, and Waisman MR750.

Rationale for the change

The ADRC standard core protocol has been in place for many years. New and updated sequences are being implemented that take advantage of new technology and techniques and will help our center stay at the forefront of imaging research in AD and related disorders. The focus remains on anatomic imaging, diffusion imaging and vascular health imaging.

What’s new?

The T1-weighted scan has been changed from a BRAVO to a modified ADNI3 T1-w. This will ensure compatibility with all the ADRCs nationwide and other national/international studies.

PCVIPR has been modified to be shorter. Continues to rely on cardiac gating.

The NODDI DTI scan has been replaced by 2 “HYDI” DTI scans. The HYDI scans must be run in tandem (acquiring only one is not useful).

ASL has been modified to accommodate variability in labeled blood arrival time into the imaging field of view. This multi-delay ASL sequence is called UW eASL. Arrival time ‘delay’ corrected maps of cerebral blood flow are automatically created.

A short CUBE T2 has been added to the end of the protocol. This cardiac-gated sequence improves anatomic segmentation in post-processing pipelines. There are 4 versions of the sequence, choose the version based on the participant’s heart rate just prior to the scanning of this sequence.  There’s an “ungated” version to be used when the pulse ox is not working well.

P-files will no longer be generated for PCVIPR and MPnRAGE.  You’ll notice that spots for recording those file names have been removed from the flowsheets. Scan archive files are created instead and these will be automatically saved and transferred.

What’s the same?

We will continue to collect MPnRAGE, CUBE T2 FLAIR, pseudoswan (SWI), and DWI.

Utilizing the new protocol

This core set of sequences will be used in full by ADRC Clinical Core and WRAP.  The ADRC Biomarker core will provide analytic support for this protocol. The core will not be able to support protocols that deviate from this standard ADRC protocol.

We strongly encourage existing projects to adopt this protocol as soon as feasible. All new ADRC-linked projects should use a minimum of the T1-w and T2 FLAIR sequences (together with the DWI and SWI) from this standard ADRC Core protocol in order to maximize ability to combine data across projects.

Individual study teams will be responsible for coordinating their transitions, and may copy the sequences they plan to adopt from the ADRC_Core protocol saved on the relevant scanner.

*New projects should contact Caitlin to discuss logistics of using the sequences*

New MRI scanner

There’s a new MRI scanner in the mix – the WIMR1 Premier, the newer 3T from GE.  When to use this new scanner:

-New ADRC participants may be scanned on the Premier.  ADRC participants who’ve already had MRI(s) on the existing WIMR 3T scanner (the WIMR2 MR750) should continue to be scanned on that scanner for consistency.

-Any WRAP participant may be scanned on any 3T scanner.

-Scanner should be kept consistent for each participant going forward (if a participant is scanned on the Premier in 2018, they should be scanned on the Premier at subsequent study visits too).  Scanner will now be tracked in PANDA and REGGIE for future reference when scheduling follow-up visits.

Fasting requirement

Continue to ask participants to refrain from caffeine and nicotine for 4 hours and 2 hours respectively. The MRI protocol no longer requires fasting from food.

OTC medications and prescribed medications should be taken as usual (However, LP fasting rules supersede if LP is scheduled the same day as MRI).

New flowsheets are available:

Save your flowsheets in a location where study staff can easily access them.  Also available on this KB page: https://kb.wisc.edu/warp/internal/page.php?id=86917

Guidance for what to do in the case of participant claustrophobia, sedative use, and running out of time is included within the flowsheet.

Finalized flowsheets are available for ADRC Clinical Core and WRAP; other studies should utilize these as templates or sources of information, but are responsible for editing their own flowsheets.

Note that sequence times differ between the MR750 and Premier.  Make sure to use the correct flowsheet.

Regulatory implications

Non-Significant Risk (NSR) letters: An NSR letter for UW eASL will need to be added to your study application before running the revised sequence. Other current NSR letters (for MPnRAGE, PCVIPR, and possibly HYDI DTI), are written broadly enough that they cover these sequence parameter changes.  Consult with your study PI if you have any questions about your NSR letters.  The sequences that are modified (not “product”) include:

MPnRAGE, PCVIPR, UW eASL, HYDI’s

Fasting: each PI and study should decide how to handle the slightly revised fasting suggestions.  A change in the requirement will likely necessitate an IRB change (likely an expedited change of protocol).

SEND QUESTIONS OR PROBLEMS TO CAITLIN CLEARY, Biomarker Core Manager

ccleary@wisc.edu  (608) 262-9620